ECE528L

Robotics and Embedded Systems

# FPV RC Car

Edgar Gutierrez

Instructor: Aaron Nanas

Fall 2025

## Introduction

## This project focused on the design and implementation of a First-Person View (FPV) Remote-Controlled (RC) Car that integrates embedded systems concepts such as PWM-based motor control, wireless communication, and real-time video transmission. The primary objective was to retrofit a toy-grade RC chassis (DEERC) with a modern microcontroller (Seeed Studio XIAO ESP32S3 Sense) to enable control via a web interface hosted on the device itself.

The project demonstrates key course topics including:

- Timers and PWM: Utilizing hardware timers (LEDC peripheral) to generate precise Pulse Width Modulation signals for speed control.

- GPIO Manipulation: Configuring pins for digital I/O to drive H-Bridge logic.

- Embedded Web Server: Establishing a Wi-Fi Access Point and hosting an asynchronous web server to handle user input and MJPEG video streaming concurrently.

Hardware

| Description | Quantity | Manufacturer |
| --- | --- | --- |
| Seeed Studio XIAO ESP32S3 Sense | 1 | Seeed Studio |
| BTS7960 43A Motor Driver | 1 | Teyleten Robot |
| DRV8833 Motor Driver | 1 | Koobok |
| DEERC Remote Control Car | 1 | DEERC |
| 7.4V Li-Ion Battery 1500mAh | 1 | DEERC |
| LM2596 DC-DC Buck Converter | 1 | AITRIP |
| Lever Nuts | 2 | WAGO |
| Jumper Wires | X | Generic |

Analysis and Results

System Architecture: The system architecture was designed around the XIAO ESP32S3, which serves as the central processing unit. The microcontroller was configured in Wi-Fi Access Point (AP) mode, broadcasting a network named FPV_RC_CAR. This allowed a client device (iPad/Smartphone) to connect directly without external routers, ensuring low latency.

Power Distribution: A critical engineering challenge was managing the different voltage requirements of the components. A Hybrid Power Rail was implemented:

- High Voltage Rail (7.4V): The Li-Ion battery connects directly to the BTS7960 driver to supply high current (up to 10A peak) to the parallel-wired front and rear drive motors.

- Regulated Logic Rail (5.0V): An LM2596 Buck Converter steps down the battery voltage to a stable 5.0V. This rail powers the XIAO ESP32S3 and the logic circuits of the motor drivers, preventing "Brownout" resets during high-load motor startups.

Motor Control Implementation (PWM & GPIO): The software utilizes the LEDC peripheral (ESP32's hardware timer) to generate PWM signals.

- Drive System: The rear and front motors are wired in parallel to a single BTS7960 driver. The PWM frequency was set to 1000 Hz with 8-bit resolution (0-255), allowing smooth acceleration control via the web interface slider.

- Steering System: The stock 5-wire servo was reverse-engineered as a standard DC motor. It is driven by a DRV8833 driver using "Bang-Bang" control logic (Full Left / Full Right / Center), as the sensor feedback wires were bypassed for this implementation.

Web Interface & FPV Streaming: A custom HTML5/JavaScript interface was stored in the microcontroller's program memory (PROGMEM). It features a dual-joystick layout for intuitive control. The video stream is handled by capturing JPEG frames from the OV3660 camera and streaming them via HTTP. Latency was optimized by setting the camera to CAMERA_GRAB_LATEST mode and reducing resolution to QVGA (320x240), achieving a smooth frame rate suitable for FPV driving.

Conclusion

In this project, we successfully designed and built a functional FPV RC car that meets all initial requirements. We learned how to interface high-current inductive loads (motors) with sensitive logic circuits using H-Bridge drivers and voltage isolation. The project reinforced our understanding of real-time control systems, specifically the relationship between Timer registers and PWM duty cycles. By integrating a web server directly onto the microcontroller, we demonstrated the capabilities of modern IoT devices in robotics. The final system achieves a control latency of under 100ms and provides a stable video feed, proving the viability of the ESP32S3 for low-cost telepresence robotics.

Software

| Tool / Library | Purpose |
| --- | --- |
| Arduino IDE / ESP-IDF | Programming and flashing firmware |
| ESPAsyncWebServer / WiFi.h | Create web interface for live control |
| PWM / LEDC | Motor and servo speed control |
| esp_camera.h Library | Handle OV3660 camera and streaming |
| HTML, CSS3 + JavaScript UI | Browser-based control dashboard |
| WiFi.h Library | library for ESP32 is used to manage the Wi-Fi connection |

Block Diagram

Pinout Plan

| XIAO ESP32-S3 | Connection |
| --- | --- |
| D0 | BTS7960(RWPM) |
| D1 | BTS7960(LWPM) |
| D2 | DRV8833(IN1) |
| D3 | DRV8833(IN2) |
| D4 | Not Connected (N/C) |
| D5 | Not Connected (N/C) |
| D6 | Not Connected (N/C) |
| D7 | Not Connected (N/C) |
| D8 | Not Connected (N/C) |
| D9 | Not Connected (N/C) |
| D10 | Not Connected (N/C) |
| 3V3 | BTS7960(R_EN), BTS7960(L_EN), DRV8833(EEP) |
| GND | Common Ground |
| 5V | LM2596(OUT+) |

| BTS7960 Driver(Motors) | Connection |
| --- | --- |
| B+ | Battery +7.4V |
| B- | Battery GND |
| M+ | Front/Rear Motors + |
| M- | Front/Rear Motors - |
| VCC | LM2596 OUT+ (5V) |
| GND | Common Ground |
| R_IS | Not Connected (N/C) |
| L_IS | Not Connected (N/C) |
| R_EN | XIAO 3.3V |
| L_EN | XIAO 3.3V |
| RPWM | XIAO D0 |
| LPWM | XIAO D1 |

| DRV8833 Driver (Steering Motor) | Connection |
| --- | --- |
| VCC/VM | LM2596 OUT+ (5V) |
| GND | Common Ground |
| IN1 | XIAO D2 |
| IN2 | XIAO D3 |
| IN3 | Not Connected (N/C) |
| IN4 | Not Connected (N/C) |
| EEP | XIAO 3.3V |
| OUT1 | Servo Yellow Wire |
| OUT2 | Servo Brown Wire |

| LM2596 (Voltage Regulator) | Connection |
| --- | --- |
| IN+ | Battery Red Wire (+7.4V) |
| IN- | Battery Black Wire (GND) |
| OUT+ | 5V Rail (Powers XIAO, BTS7960 Logic, DRV8833) |
| OUT- | Common Ground |